Home Automation Dashboard with ESP12 and Raspberry Pi

This document summarizes the complete setup of a Home Automation Dashboard using ESP12 (NodeMCU), Raspberry Pi, MQTT, Flask, and Ngrok, along with debugging steps and solutions. A visual diagram of the system is included for easy reference.

1. Hardware & Tools

- ESP12 / NodeMCU: Controls the LED

- Raspberry Pi: Runs Flask dashboard and MQTT broker

- LED or Relay: Connected to ESP12

- MQTT Broker: Mosquitto running on Raspberry Pi

- Software:

- Arduino IDE

- Python 3.11

- Flask + Flask-SocketIO

- Paho-MQTT

- Ngrok v3

2. ESP12 Setup in Arduino IDE

Steps: 1. Installed ESP12 board in Arduino IDE via Board Manager. 2. Selected correct COM port and board. 3. Tested LED blink example. 4. Added Wi-Fi connection code (SSID & password). 5. Added MQTT client using PubSubClient.

MQTT Topics: - home/esp12/led/set → receives ON/OFF commands - home/esp12/led/state → publishes LED status

Debug Notes: - Compilation error PubSubClient.h → installed library. - Wi-Fi issues → verified SSID/password. - MQTT rc=-2 → checked broker IP and port.

3. Raspberry Pi MQTT Broker Setup

Steps: 1. Installed Mosquitto:

sudo apt install mosquitto mosquitto-clients
sudo systemctl enable mosquitto
sudo systemctl start mosquitto

- Tested MQTT topics via terminal.

Debug Notes: - Mosquitto not starting → restarted service, checked logs. - MQTT connection refused → verified topics and ports.

4. Flask Dashboard Setup

Steps: 1. Installed Python libraries:

pip3 install flask paho-mqtt flask-socketio

- Created Flask app with:

- / → Dashboard page

- /on and /off → MQTT commands

- Flask-SocketIO for real-time LED status

- HTML dashboard with ON/OFF buttons and status display.

Debug Notes: - LED status “Unknown” → Eventlet issue, replaced with async_mode='threading' - Flask host changed to 0.0.0.0 for external access. - Firewall port 5000 opened.

5. Local Testing

- Dashboard accessible at http://192.168.1.12:5000

- Verified LED control and real-time status updates.

- External access on same Wi-Fi works after opening firewall.

6. Remote Access via Ngrok

Steps: 1. Installed latest Ngrok v3 (ARM version for Raspberry Pi). 2. Added authtoken via command or ~/.ngrok2/ngrok.yml. 3. Started tunnel:

ngrok http 5000

- Public URL example: https://feastful-numerously-lindsay.ngrok-free.dev

Debug Notes: - Old ngrok v2 removed. - Flask connection refused → host set to 0.0.0.0:5000

7. Remote Testing

- Open public URL on any device.

- Verified LED control and real-time updates.

- Used ngrok web interface (http://127.0.0.1:4040) for monitoring.

8. Key Lessons / Notes

- Flask host must be 0.0.0.0 for ngrok.

- Eventlet deprecated → use async_mode='threading'.

- MQTT topics must match on ESP and Flask.

- Ngrok free plan → URLs change on each restart.

- Firewall port 5000 must be open.

- Use ngrok web interface for debugging.

9. Outcome

- ESP12 LED controlled via Flask dashboard.

- Real-time LED status via MQTT + Flask-SocketIO.

- Dashboard accessible locally and remotely.

- Setup ready for expansion to multiple devices.

10. System Diagram

Home Automation Diagram

Home Automation Diagram

Flow Summary: 1. ESP12 connects to Wi-Fi, subscribes/publishes MQTT topics. 2. Raspberry Pi runs Mosquitto broker. 3. Flask + SocketIO listens to MQTT and updates dashboard. 4. Local users access dashboard via http://<pi_ip>:5000. 5. Remote users access dashboard via Ngrok public URL. 6. LED control and status updates work from anywhere.

This document provides a complete reference of the setup, debugging steps, and final working solution for your Home Automation dashboard project.

Command Used:

1  sudo apt update

2  sudo apt-get update

3  ping -c 4 google.com

4  sudo apt update

5  sudo nano /etc/apt/sources.list

6  sudo apt-get update

7  sudo apt update

8  sudo nano /etc/apt/sources.list

9  sudo apt update

10  sudo apt list --u[gradable

11  sudo apt list --upgradable

12  sudo apt-get update

13  sudo apt update

14  sudo curl http://archive.raspberrypi.com/debian/dists/bookworm/InRelease

15  cat /etc/resolv.conf

16  sudo apt full-upgrade

17  sudo apt-get update

18  sudo apt update

19  sudo apt install raspberrypi-kernel-headers

20  sudo reboot

21  vncserver-virtual

22  pip3 install fastapi uvicorn[standard] rpi.gpio

23  sudo apt-get install python3 python3-pip

24  pip3 install fastapi uvicorn[standard] rpi.gpio

25  pip3 apt install fastapi uvicorn[standard] rpi.gpio

26  python3-pip apt install fastapi uvicorn[standard] rpi.gpio

27  apt install python3-pip fastapi uvicorn[standard] rpi.gpio

28  sudo apt install fastapi uvicorn[standard] rpi.gpio

29  sudo apt full-update

30  sudo apt-get full-update

31  pip install fastapi

32  python -V

33  pip -V

34  python3 -V

35  pip3 -V

36  pip3 install fastapi[all]

37  ~/.config/pip/pip.cpnf

38  ~/.config/pip/pip.conf

39  ~/ .config/pip/pip.conf

40  python3 -m venv myenv

41  source myenv/bin/activate

42  pip install fastapi[all]

43  pip3 install fastapi uvicorn[standard] rpi.gpio

44  deactivate

45  ls

46  mkdir fastapi

47  cd fastapi

48  sudo nano main.py

49  ls

50  cat main.py

51  uvicorn -V

52  uvicorn main:app --reload

53  source myenv/bin/activate

54  python3 -m venv myenv

55  source myenv/bin/activate

56  pip3 install uvicorn[standard]

57  uvicorn main:app --reload

58  pip3 install fastapi[all]

59  fastapi -V

60  pip3 install uvicorn

61  uvicorn main:app --host 0.0.0.0 --port 8000 --reload

62  sudo apt install python3-rpi.gpio

63  deactivate

64  uvicorn main:app --host 0.0.0.0 --port 8000 --reload

65  python -m uvicorn main:app --host 0.0.0.0 --port 8000 --reload

66  python -m uvicorn main:app --reload

67  cd ..

68  ls

69  nano ~/.bashrc

70  uvicorn --version

71  source ~/.bash_profile

72  source ~/.bashrc

73  uvicorn --version

74  uvicorn main:app --host 0.0.0.0 --port 8000 --reload

75  ls

76  cd fastapi

77  uvicorn main:app --host 0.0.0.0 --port 8000 --reload

78  uvicorn main:app --reload

79  nano main.py

80  vim main.py

81  vi main.py

82  sudo nano main.py

83  uvicorn main:app --reload

84  sudo nano main.py

85  uvicorn main:app --reload

86  sudo nano main.py

87  uvicorn main:app --reload

88  sudo nano main.py

89  uvicorn main:app --reload

90  ifconfig

91  uvicorn main:app --reload

92  sudo nano main.py

93  uvicorn main:app --reload

94  history

95  history > log.txt

96  ls

97  cat log.txt

98  shutdown

99  reboot 0

100  ls

101  sudo shutdown -h -P now

102  ls

103  cd fastapi

104  ls

105  cat log.txt

106  uvicorn main:app --reload

107  vncserver-virtual

108  sudo shutdown -h -P now

109  vncserver-virtual

110  ls

111  cd fastapi

112  ls

113  cat log.txt

114  ls

115  python led.py

116  cat led.py

117  sudo nano led.py

118  uvicorn main:app --reload

119  sudo shutdown -h -P now

120  cd fastapi

121  ls

122  uvicorn main:app --reload

123  sudo systemctl status ssh

124  sudo raspi-config

125  sudo reboot

126  cat /etc/passwd

127  sudo cut -d: f1 /etc/passwd

128  cut -d: f1 /etc/passwd

129  who

130  whoami

131  sudo getent group sudo

132  sudo adduser pi

133  who

134  cat /etc/passwd

135  hostname -I

136  ip addr show

137  ssh pi@192.168.31.203

138  ifconfig

139  hostname -I

140  sudo systemctl status ssh

141  sudo systemctl enable ssh

142  sudo systemctl start ssh

143  sudo ufw allow 22/tcp

144  sudo ufw reload

145  sudo ufw status

146  ifconfig

147  sudo apt update

148  sudo apt install telnet -y

149  sudo sytemctl restart mosquitto

150  sudo systemctl restart mosquitto

151  sudo shutdown

152  sudo netstat -tuln | grep 1883

153  sudo nano /etc/mosquitto/mosquitto.conf

154  sudo systemctl restart mosquitto

155  sudo systemctl status mosquitto

156  sudo systemctl restart mosquitto

157  sudo nano /etc/mosquitto/mosquitto.conf

158  sudo systemctl restart mosquitto

159  sudo nano /etc/mosquitto/mosquitto.conf

160  sudo systemctl status mosquitto.service

161  journalctl -xe

162  sudo nano /etc/mosquitto/mosquitto.conf

163  sudo nano /etc/mosquitto/conf.d/

164  sudo nano /etc/mosquitto/conf.d

165  sudo mosquitto -c /etc/mosquitto/mosquitto.conf -v

166  sudo nano /etc/mosquitto/mosquitto.conf

167  sudo mosquitto -c /etc/mosquitto/mosquitto.conf -v

168  sudo systemctl restart mosquitto

169  sudo systemctl status mosquitto

170  sudo netstat -tuln | grep 1883

171  mosquitto_pub -h 192.168.31.203 -t "home/bulb" -m "ON"

172  mosquitto_pub -h 192.168.31.203 -t "home/bulb" -m "OFF"

173  mosquitto_pub -h 192.168.31.203 -t "home/bulb" -m "ON"

174  mosquitto_pub -h 192.168.31.203 -t "home/fan" -m "ON"

175  mosquitto_pub -h 192.168.31.203 -t "home/fan" -m "OFF"

176  sudo apt update && sudo apt upgrade -y

177  sudo reboot

178  docker logs -f homeassistant

179  curl http://localhost:8123

180  vim /home/pi/homeassistant/config/configuration.yaml

181  vi /home/pi/homeassistant/config/configuration.yaml

182  sudo nano /home/pi/homeassistant/config/configuration.yaml

183  docker restart homeassistant

184  docker logs homeassistant 2>&1 | grep "Home Assistant"

185  curl http://localhost:8123

186  docker logs homeassistant 2>&1 | grep "Home Assistant"

187  sudo apt update

188  sudo apt install -y python3 python3-venv python3-pip libffi-dev libssl-dev autoconf build-essential

189  timedatectl list-timezones

190  sudo timedatectl set-timezone Asia/Kolkata

191  docker rm -f homeassistant

192  mkdir -p ~/homeassistant

193  docker run -d   --name homeassistant   --privileged   --restart=unless-stopped   -e TZ=Asia/Kolkata   -v /home/pi/homeassistant:/config   --network=host   ghcr.io/home-assistant/home-assistant:stable

194  docker rm -f homeassistant

195  mkdir -p ~/homeassistant

196  docker run -d   --name homeassistant   --privileged   --restart=unless-stopped   -e TZ=Asia/Kolkata   -v /home/pi/homeassistant:/config   --network=host   ghcr.io/home-assistant/home-assistant:stable

197  docker logs -f homeassistant

198  wget -O - https://get.hacs.xyz | bash -

199  sudo nano /home/pi/homeassistant/config/configuration.yaml

200  sudo docker exec -it homeassistant /bin/bash

201  wget -O - https://get.hacs.xyz | bash -

202  sudo docker exec -it homeassistant /bin/bash

203  sudo docker restart homeassistant

204  ls

205  cd homeassistant

206  sudo docker exec -it homeassistant /bin/bash

207  cd ..

208  sudo docker exec -it homeassistant /bin/bash

209  sudo docker restart homeassistant

210  sudo reboot

211  sudo systemctl status mosquitto

212  mosquitto_sub -h localhost -t test/topic

213  mosquitto_sub -h localhost -t home/test

214  mosquitto_sub -h 192.168.1.12 -t home/test

215  mosquitto_sub -h localhost -t home/esp12/led/state

216  ls

217  cd mqtt_dashboard

218  ls

219  python3 app.py